2_ Implementation of BGP using AS_path attribute.

Aim: Implementation of BGP using AS_path attribute.

Take 3 routers -> Configure -> slots -> NM-4T

On Router console type following commands one by one.

R1 Console

conf t

int s1/0

ip add 192.168.1.1 255.255.255.0

no sh

int s1/1

ip add 172.24.1.1 255.255.255.0

no sh

R2 Console

conf t

int s1/0

ip add 192.168.1.2 255.255.255.0

no sh

R3 Console

conf t

int s1/1

ip add 172.24.1.3 255.255.255.0

no sh

To add loopback address ,On Router console type following commands one by one.

R1 Console

int lo0

ip add 10.1.1.1 255.255.255.0

R2 Console

int lo0

ip add 10.2.2.2 255.255.255.0

R3 Console

int lo0

ip add 10.3.3.3 255.255.255.0

To add bgp protocol, On Router console type following commands one by one.

R1 Console

router bgp 100

neighbor 192.168.1.2 remote-as 200

neighbor 172.24.1.3 remote-as 300

network 10.1.1.0 mask 255.255.255.0

R2 Console

router bgp 200

neighbor 192.168.1.1 remote-as 100

network 10.2.2.0 mask 255.255.255.0

R3 Console

router bgp 300

neighbor 172.24.1.1 remote-as 100

network 10.3.3.0 mask 255.255.255.0

To show ip route type following command in each router console

do sh ip route

To verify output type following commands: (OUTPUT)

do ping 10.3.3.3 source lo0

do ping 10.2.2.2 source lo0