6_ Demonstrate inter vlan routing.

AIM: Demonstrate inter vlan routing.

# Task1: check VLAN config in each switch type command for all switches:

en

show vlan br

CHECK IF ALL SWITCHES HAVE SAME VLAN (1002,1003,1004,1005...)

# Task2: disable all ports on all the switches commands for all switches:

conf t

interface range fa0/1-24 shutdown

interface range gi0/1-2 shutdown

# Task3: Perform basic switch configurations like assign name to switches, password to switches as well as gateways.

commands for all switches: exit

(config)

hostname s0 enable secret class

no ip domain-lookup

ip default-gateway 172.17.99.1

line console 0 (config-line) password cisco login

line vty 0 15 password cisco login

end

# Task4: On the interfaces of the switch 2 connect it to the PCs, configure access mode and enable

commands for s2:

(config) int fa0/11 (config-if)

switchport mode access no shutdown

int fa0/12

switchport mode access no shutdown

int fa0/13

switchport mode access no shutdown

# Task5: Configure IP addresses on the three PCs and the server

PC0-> Desktop -> IP config IP: 172.17.10.21 255.255.255.0

Default gateway: 172.17.10.1

PC1-> Desktop -> IP config IP: 172.17.20.22 255.255.255.0

Default gateway: 172.17.20.1

PC2-> Desktop -> IP config IP: 172.17.30.23 255.255.255.0

Default gateway: 172. 17.30.1

Server -> Desktop -> IP config IP: 172.17.50.254 255.255.255.0

Default gateway: 172.17.50.1

# Task6: Configure VTP protocol on the switches. s0 will be VTP server, s1 & s2 will be VTP client

s0:

Password: class en

(#)

Password: conf t (config)

vtp mode server vtp domain vsit vtp password cisco s1:

Password:

en #

Password: conf t (config)

vtp mode client vtp domain vsit vtp password cisco s2:

Password:

en #

Password: conf t (config)

vtp mode client vtp domain vsit vtp password cisco

# Task7: Configure trunking codes on all connections between switches and enable them

s0:

(config)

int range fa0/1-3 (config-if)

switchport mode trunk switchport trunk native vlan 99 no shutdown

int range fa0/5-6 switchport mode trunk

switchport trunk native vlan 99 no shutdown

s2:

(config)

int range fa0/3-6 (config-if)

switchport mode trunk switchport trunk native vlan 99 no sh

s1:

(config)

int range fa0/1-4 (config-if)

switchport mode trunk switchport trunk native vlan 99

no sh

s0:

(config-if-range) exit

(config) vlan 99

name management (config-vlan)

vlan 10 name staff vlan 20

name students vlan 30

name guests

exit

do sh vlan br (On s0 and s2)

# Task8: Configure interface vlan 99 on all the switches

s0:

(config) int vlan 99 (config-if)

ip add 172.17.99.11 255.255.255.0

end

s2:

(config) int vlan 99 (config-if)

ip add 172.17.99.12 255.255.255.0

end

s1:

(config) int vlan 99 (config-if)

ip add 172.17.99.13 255.255.255.0

end

# Task9: Configure vlan 10, vlan 20 and vlan 30 on switch 2

s2:

(config) int fa0/11 (config-if)

switchport access vlan 10 int fa0/12

switchport access vlan 20 int fa0/13

switchport access vlan 30

# Task10: perform configuration on router

Router:

en conf t

hostname r1

no ip domain-lookup line console 0 (config-line) password cisco

login

line vty 0 15 password cisco login

end

conf t (config)

enable secret class int fa0/1

no sh

int fa0/1.1 (config-subif)

encapsulation dot1q 1

ip add 172.17.1.1 255.255.255.0

int fa0/1.10 encapsulation dot1q 10

ip add 172.17.10.1 255.255.255.0

int fa0/1.20 encapsulation dot1q 20

ip add 172.17.20.1 255.255.255.0

int fa0/1.30 encapsulation dot1q 30

ip add 172.17.30.1 255.255.255.0

int fa0/1.99

encapsulation dot1q 99 native

ip add 172.17.99.1 255.255.255.0